# Advanced HB

- Device should support up to 5 types of HB packets for each monitor port in each segment.

- Segment should switch to expired state if number of working segment decreased below the minimum working number (selected by command)

- The segment should restore inline state when the number of working members reaches the minimum working number.

- Segment should be able to return to inline mode manually or automatically.

- Each HB packet for each segment can have different content except MAC addresses.

- HB packet mode:

- HB send from MON0, and receive response by MON1

- HB send from MON1, and receive response by MON0

- Bidirectional: send from each port and receive response from the counterpart port

- Segment will switch to expired state if one of direction will be failed (selected by command)

- Segment will switch to expired state if both of direction will be failed (selected by command)

- Response content must be the same for any type of inline application processing.

- New and old CLI commands needed for multiple HB-type feature (see in appendix A commands help and examples):

- del_hb				- delete HB packet

- set/get_hb_params 		- set/get HB packet type and other parameters

- set/get_hb_state			- set/get HB packet state

- set/get_min_work_members	- set/get minimal number of HB-types working members for segment

- load_hb_pkt			- load HB packet content, should be compatible with HB type.

- get_hb_pkt			- display HB packet

- get/set_hb_src_mac		- set HB packet source MAC for current segment

- get/set_hb_dst_mac		- set HB packet destination MAC for current segment

- set_default_hb_mac		- set HB packet default source and destination MAC for current segment

- set_default_hb_pkt		- set default HB for current segment

- IP used in HB packets must not be used in any other traffic.

- MAC-type HB packet is not compatible with IP-type and VLAN_IP-type HB packets.

- Only one MAC-type HB packet can exist for one segment.

- Each IP/VALN_IP-type packet should have source and/or destination IP different from other packets.

- HB packets install order:

- del_hb (used only if needed to prevent using incompatible HB types.

- load_hb_pkt

- set_hb_params

- HB packet can be entered by binary or text file

- HB packet file name must contain prefix “hb_” and suffix “bin” or “txt”

- HB packet file name can contain 8 – 20 characters including prefix and suffix.

Allowed following characters:

A-Z, a-Z, 0-9, '_', '.', '-', '@', ':', '>', '+', '=', ','

- HB packet size:

- Binary file: 	24-1024

- Text file: 	72-3454

- HB packet file should not contain HB packet checksum

- HB packet text file example:

00 50 c2 3c  60 00 00 50    c2 3c 60 01  81 37 ff ff

00 30 00 00  00 00 40 04    ec a2 c6 13  01 02 c6 13

01 01 00 00  00 00 00 00    00 00 00 00  01 02 03 04

05 06 07 08  09 0a 0b 0c    0d 0e 0f 00  10 20 30 40

65 66 67 66

- Adding new HB packets feature will decrease filters number from 244 to 396

# HB packets type

## MAC HB packet

MAC HB packets have different MAC addresses for each segment.

MAC addresses can be changed by command

set_hb_dst_mac: MAC1

set_hb_src_mac: MAC2

HB packet that will be send from MON0 port

| Destination MAC | Source MAC | payload |
| --- | --- | --- |
| MAC1 | MAC2 |  |

Response:

| Destination MAC | Source MAC | payload |
| --- | --- | --- |
| MAC2/MAC1 | MAC1/MAC2 |  |
| Used for detecting HB |  |  |

HB packet that will be send from MON1 port

| Destination MAC | Source MAC | payload |
| --- | --- | --- |
| MAC2 | MAC1 |  |

Response:

| Destination MAC | Source MAC | payload |
| --- | --- | --- |
| MAC1/MAC2 | MAC2/MAC1 |  |
| Used for detecting HB |  |  |

## IP HB packet

MAC HB packets have different MAC addresses for each segment.

IP HB packets have different IP addresses for HB type in segment.

MAC addresses can be changed by command

set_hb_dst_mac: MAC1

set_hb_src_mac: MAC2

set_hb_dst_ip: IP1

set_hb_src_ip: IP2

HB packet that will be send from MON0 port

| Destination MAC | Source MAC | type | IP header |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Source IP | Destination IP |  |  |
| MAC1 | MAC2 | 08 00 |  | IP2 | IP1 |  |  |

Response:

| Destination MAC | Source MAC | type | IP header |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Source IP | Destination IP |  |  |
| MAC2 | MAC1 | 08 00 |  | IP1/IP2 | IP2/IP1 |  |  |
|  |  |  |  | Used for detecting HB |  |  |  |

HB packet that will be send from MON1 port

| Destination MAC | Source MAC | type | IP header |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Source IP | Destination IP |  |  |
| MAC2 | MAC1 | 08 00 |  | IP1 | IP2 |  |  |

Response:

| Destination MAC | Source MAC | type | IP header |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Source IP | Destination IP |  |  |
| MAC1 | MAC2 | 08 00 |  | IP2/IP1 | IP1/IP2 |  |  |
|  |  |  |  | Used for detecting HB |  |  |  |

## IP with VLAN HB packet

MAC HB packets have different MAC addresses for each segment.

IP HB packets have different IP addresses for HB type in segment.

MAC addresses can be changed by command

set_hb_dst_mac: MAC1

set_hb_src_mac: MAC2

set_hb_dst_ip: IP1

set_hb_src_ip: IP2

set_hb_vlan: VLAN (should not be internal Broadcom VLAN – default: 1, can be changed by set_int_vlan)

HB packet that will be send from MON0 port

| Destination MAC | Source MAC | 802.1Q |  |  |  | type | IP header |  |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Source IP | Destination IP |  |  |  |
|  |  | 8100 |  |  | VID | 08 00 |  |  |  |  |  |  |
| MAC1 | MAC2 |  |  |  | VLAN |  |  | IP2/IP1 | IP1/IP2 |  |  |  |

Response:

| Destination MAC | Source MAC | 802.1Q |  |  |  | type | IP header |  |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Source IP | Destination IP |  |  |  |
|  |  | 8100 |  |  | VID | 08 00 |  |  |  |  |  |  |
| MAC2 | MAC1 |  |  |  | VLAN |  |  | IP1/IP2 | IP2/IP1 |  |  |  |
|  |  |  |  |  |  |  |  | Used for detecting HB |  |  |  |  |

HB packet that will be send from MON1 port

| Destination MAC | Source MAC | 802.1Q |  |  |  | type | IP header |  |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Source IP | Destination IP |  |  |  |
|  |  | 8100 |  |  | VID | 08 00 |  |  |  |  |  |  |
| MAC2 | MAC1 |  |  |  | VLAN |  |  | IP1 | IP2 |  |  |  |

Response:

| Destination MAC | Source MAC | 802.1Q |  |  |  | type | IP header |  |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Source IP | Destination IP |  |  |  |
|  |  | 8100 |  |  | VID | 08 00 |  |  |  |  |  |  |
| MAC1 | MAC2 |  |  |  | VLAN |  |  | IP2 | IP1 |  |  |  |
|  |  |  |  |  |  |  |  | Used for detecting HB |  |  |  |  |

Appendix A

Note 1: Almost each command contains [module:segment[hb_id]] | [hb_id] parameter:

- If module:segment was entered, but hb_id parameter was not – command work with first HB packet for specified segment.

- If module:segment:hb_id parameter was entered – command work with specified HB packet for specified segment.

- If only hb_id parameter was entered – command work with specified HB packet for                              current segment.

Note 2: Swap parameters mean that this parameter is swapped by user application when send from MON0 or MON1 ports.

del_hb – delete one HB type

del_hb  <[module:segment:hb_id] | [hb_id]>

- delete HB for current or specified segment.

hb_id - HB type index (1 - 5).

If module:segment:hb_id parameter was

entered - delete the specified HB

of the specified segment.

If only hb_id parameter was entered - delete

the specified HB of the current segment.

Examples:

del_hb 1

del_hb 1:1:3

set_hb_params   - set HB type and parameters

set_hb_params <[module:segment[:hb_id]] | [hb_id]>

<mac m0_swap_src_mac m0_swap_dst_mac m1_swap_src_mac m1_swap_dst_mac>

<ip src_ip dst_ip m0_swap_src_ip m0_swap_dst_ip m1_swap_src_ip m1_swap_dst_ip>

<valn_ip vlan src_ip dst_ip m0_swap_src_ip m0_swap_dst_ip m1_swap_src_ip

m1_swap_dst_ip>

- set HB parameters for the current or specified

segment.

hb_id - HB type index (1 - 5).

m0/m1_swap_src_mac (on|off) - set the location

of src MAC in the RSP packet on mon1/mon0.

m0/m1_swap_dst_mac (on|off) - set the location

of dst MAC in the RSP packet on mon1/mon0.

vlan - HB vlan number,

src_ip - HB source IP,

dst_ip - HB destination IP,

m0/m1_swap_src_ip (on|off) - set the location

of src IP in the RSP packet on mon1/mon0.

m0/m1_swap_dst_ip (on|off) - set the location

of dst IP in the RSP packet on mon1/mon0.

Examples:

set_hb_params 1 mac off off off off

set_hb_params 2 mac off on off on

set_hb_params 1 ip 192.168.1.0 193.168.1.2 off off off off

set_hb_params 2 ip 192.168.2.0 193.168.2.2 off off off off

set_hb_params 2 vlan_ip 12 192.168.2.0 193.168.2.2 off off off off

set_hb_params 1 vlan_ip 12 192.168.2.0 193.168.2.2 off off off off

set_hb_params 3 ip 192.168.3.0 193.168.3.2 off off off off

set_hb_params 4 ip 192.168.4.0 193.168.4.2 off off off off

set_hb_params 5 ip 192.168.5.0 193.168.5.2 off off off off

get_hb_params  - display parameters for HB packet

get_hb_params [module:segment[:hb_id]] | [hb_id]

- show HB parameters for the current or specified

segment.

hb_id - HB type index (1 - 5).

If module:segment was entered, but hb_id

parameter was not - will show parameters

of the first HB of the specified segment.

If module:segment:hb_id parameter was

entered - will show parameters of the

specified HB of the specified segment.

If only hb_id parameter was entered - will

show parameters of the specified HB

of the current segment.

Examples:

get_hb_params 1:1:1

HB 1 parameters:

state:                 loaded, configured, used, working,

type:                  VLAN_IP,

vlan:                  11,

swap mon0 src IP:      off,

swap mon0 dst IP:      off,

swap mon1 src IP:      off,

swap mon1 dst IP:      off,

source IP              192.168.11.11,

destination IP         193.168.11.12.

set_hb_state  - enable/disable HB state

set_hb_state <[module:segment[:hb_id]] | [hb_id]> on|off

- set HB state for current or specified segment.

By default, HB enabled after setting HB

parameters.

hb_id - HB type index (1 - 5).

If module:segment was entered, but hb_id

parameter was not - set the state for

the first HB of the specified segment.

If module:segment:hb_id parameter was entered -

set the state for the specified HB

of the specified segment.

If only hb_id parameter was entered - set the

state of the specified HB of the current segment.

Examples:

set_hb_state 1 off

set_hb_state 3 on

get_hb_state – display HB state

ERROR: wrong parameter value or its length ("dd")

get_hb_state [[module:segment[:hb_id]] | [hb_id]]

- show HB state for the current

or specified segment.

hb_id - HB type index (1 - 5).

If module:segment was entered, but hb_id

parameter was not - will show the state of

the first HB of the specified segment.

If module:segment:hb_id parameter was entered -

will show the state of the specified HB

of the specified segment.

If only hb_id parameter was entered - will show

the state of the specified HB

of the current segment.

Examples:

get_hb_state

HB 1 state:                 MAC , loaded, configured, used, expired.

HB 2 state:                 NONE, loaded.

HB 3 state:                 NONE.

HB 4 state:                 NONE.

HB 5 state:                 NONE, loaded.

get_hb_state 1:1

HB 1 state:                 VLAN_IP, loaded, configured, used, working.

HB 2 state:                 VLAN_IP, loaded, configured.

HB 3 state:                 VLAN_IP, loaded, configured, used, working.

HB 4 state:                 VLAN_IP, loaded, configured, used, working.

HB 5 state:                 VLAN_IP, loaded, configured, used, working.

get_hb_state 3:1

HB 1 state:                 VLAN_IP, loaded, configured.

HB 2 state:                 VLAN_IP, loaded, configured.

HB 3 state:                 VLAN_IP, loaded, configured, used, expired.

HB 4 state:                 VLAN_IP, loaded, configured, used, expired.

HB 5 state:                 VLAN_IP, loaded, configured, used, expired.

set_hb_min_work_members  -  set minimal HB working members

set_hb_min_work_members [module:segment] all|1-5

- set a minimum number of working HBs before

transitioning current or specified segment

to expired state.

all - all HBs should work.

With this value - changing the number of

HBs will automatically correct this

parameter.

1 - 5 - With these values - changing the

number of HBs will not change this parameter.

If module:segment parameter was entered -

set the number of minimum working members

for the specified segment,

otherwise - set the number of minimum working

members for the current segment.

Examples:

set_hb_min_work_members all

set_hb_min_work_members 3

get_hb_min_work_members  - displays minimal HB working members for segment

get_hb_min_work_members [module:segment]

- show a minimum number of working HBs before

transitioning current or specified segment

to expired state.

If module:segment parameter was entered -

will show the number of minimum working members

of the specified segment,

otherwise - will show the number of minimum

working members of the current segment.

Examples:

get_hb_min_work_members

HB minimum work members:    1.

load_hb_pkt – load content for HB packet

load_hb_pkt file_name [ip root] [module:segment[:hb_id]] | [hb_id]

- loads new hb packet,

file_name - 8-20 characters, prefix: hb_

file should contain HB context (ext: txt or bin)

and be in the tftp root directory.

ip    - tftp server IP address,

root  - tftp root directory.

hb_id - HB type index (1 - 5).

If module:segment parameter was not entered -

the packet will be used for the first HB (hb_id=1)

of all segments.

If module:segment parameter was entered,

but hb_id was not - the packet will be used for

the first HB (hb_id=1) of the specified segment.

If module:segment:hb_id parameter was entered -

the packet will be used for the specified hb_id

of the specified segment.

Examples:

load_hb_pkt hb_tcp.txt 192.168.0.2 tftpboot/hb_new/IP 2

load_hb_pkt hb_tcp_unix.txt 192.168.0.2 tftpboot/hb_new/IP 2

load_hb_pkt hb_tcp.bin 192.168.0.2 tftpboot/hb_new/IP 2

load_hb_pkt hb_ping_ip.txt 192.168.0.2 tftpboot/hb_new/IP 2

load_hb_pkt hb_vlan6_ip.txt 192.168.0.2 tftpboot/hb_new/IP 2

load_hb_pkt hb_64_dos.txt 192.168.0.2 tftpboot 1

load_hb_pkt hb_64_unix.txt 192.168.0.2 tftpboot 1

load_hb_pkt hb_64.bin 192.168.0.2 tftpboot 1